Connor Stage Panel
PART 1 - GENERAL 

1.1 QUALITY ASSURANCE 
A.
Manufacturer 

1. Manufacturer of flooring shall be a firm specializing in manufacturing products specified in this section.
B.
Installer (Flooring Contractor)
1.
The complete installation of the flooring as described in the scope of this specification shall be carried out
      by an experienced installer (Flooring Contractor), and the work shall be performed in accordance with most
      recent installation instructions of the manufacturer.
2.
Installer (Flooring Contractor) shall be liable for all matters related to installation for a period of one year
      after the floor has been substantially installed and completed.

1.2 SUBMITTALS 
A.
Specification – Submit stage panel specification sheets.

B.
Sample - Submit one sample of specified flooring product if requested by architect.

1.3 WORKING CONDITIONS 
A. The flooring specified herein shall not be installed until all masonry, painting, plaster, tile, marble and terrazzo work is completed, and overhead mechanical trades and painters have finished in the stage floor areas.  The building shall be enclosed and weathertite.

B. Moderate room temperature of 65 degrees (18 degrees Celsius) or more shall be maintained a week preceding and throughout the duration of the work.  Humidity conditions within the building shall approximate the humidity conditions that will prevail when the building is occupied.

C. Permanent heat, light and ventilation shall be installed and operating during and after installation, maintaining a range of temperature and humidity compatible with the expected low and high moisture content of the flooring. The wood moisture content range is determined by the flooring contactor based on the facility’s mechanical controls and/or geographical location.

D. Flooring must be stored in a dry, well-ventilated area, not in contact with masonry, to acclimate to building conditions and shall be installed at moisture content compatible with the normally expected environmental range of temperature and relative humidity achieved while the facility is occupied.

E. General Contractor shall lock floor area after floor is finished to protect the surface from construction damage. If general contractor or owner requires use of stage area after installation, he shall protect the floor by covering with non-marring kraft paper or red rosin paper with taped joints until acceptance by owner of complete stage floor.

F. Working conditions as described above shall be followed. Variations and substitutions shall be submitted for approval to the architect who shall advise Connor of the same.
1.4 HUMIDITY CONTROL 
A. Since all wood products will expand and contract as relative humidity varies, it is important to minimize extremes between low and high. Geographical regions and available mechanicals determine the typical range of temperature and humidity for each facility. Maintaining a 15% fluctuation between highest and lowest average indoor relative humidity provides limited shrinkage and growth. Facility managers should make use of available HVAC systems to prevent excessive movement of flooring.

1.5 WARRANTY 
A. Connor warrants that the materials it has supplied will be free from manufacturing defects for a period of one year. The foregoing warranty is in lieu of and excludes all other warranties not expressly set forth herein, whether express or implied in operation of law or otherwise, including, but not limited to, any implied warranties of merchantability or fitness. This warranty is expressly limited to the flooring materials (goods) supplied by Connor. This warranty does not cover floor damage caused (wholly or in part) by fire, winds, floods, moisture, other unfavorable atmospheric conditions or chemical action, nor does it apply to damage caused by ordinary wear, misuse, abuse, negligent or intentional misconduct, aging, faulty building construction, concrete slab separation, faulty or unsuitable subsurface or site preparation, settlement of the building walls or faulty or unprofessional installation of Connor flooring systems.

B. Connor shall not be liable for incidental or consequential losses, damages or expenses directly or indirectly arising from the sale, handling or use of the materials (goods) or from any other cause relating thereto, and their liability hereunder in any case is expressly limited to the replacement of materials (goods) not complying with this agreement, or at their elections, to the repayment of, or crediting buyer with, an amount equal to the purchase price of such materials (goods), whether such claims are for breach of warranty or negligence. Any claim shall be deemed waived by buyer unless submitted to Connor in writing within 30 days from the date buyer discovered, or should have discovered, any claimed breach.

PART 2 - PRODUCTS 
2.1 MATERIALS

A.
Flooring - 

1.
15/32” x 48”x96” stage panel as manufactured by Connor Sports, Amasa, MI.

2.
Optional thickness (specify above or delete) -
a. 23/32” x 48”x96” stage panel.
b. 1/4" x 48”x96” stage panel.
PART 3 - EXECUTION
3.1 INSTALLATION
A.
Flooring


1. Install stage panel over existing full plywood surface by fastening with the provided screws and locator template, ensuring that the screws heads are flush with the surrounding areas.
3.2 CLEANING 
A.
Remove excess and waste materials from the area of work.
3.3 PROTECTION

A. Install Connor Sports Berber Vinylback entrance mat at all outdoor entrances and direct access points to the stage flooring.  For custom mats with logos please contact info@connorsports.com.

